Homework 6: 100 Points

Please submit your answers in a document and upload it in Canvas by the due date.

Present your work neat, clear, organized, logical, and make your case; don’t leave it to our interpretation of your answer. There is 10% penalty per day for delay in submitting the homework.

- Exercise 4.9 a. (20 Points). Hint: Note that in a complex (imaginary) number: a=x+yi; x is a_re, y is a_im. Arithmetic density definition is on Page 308..

FLOPs Per Iteration: 4 multiply + 2 add/sub = 6

Bytes Per Iteration: (4 loads + 2 stores) × 4 Bytes = 24 Bytes

Arithmetic IntensityI: 6 FLOPs / 24 Bytes = 0.25 FLOP/byte

- Exercise 4.10 (20 Points). Hint: The purpose is to compare a vector processor with a hybrid system of a scalar processor (host) + GPU. Draw a picture to understand it better.

Vector:

Kernel Memory Time= (200 MB + 100 MB)/ 30 GB/s= 0.3 GB/ 30 GB/s= 0.01 s= 10ms

Total= 400 ms + 10 ms= 410 ms

Hybrid:

DMA Transfers= (200 MB10 GB/s + 10 ms) + (100 MB10 GB/s + 10ms)= 50 ms

Kernel Memory Time= 200 MB + 100 MB150 GB/s= 2 ms

Total= 400 ms + 50 ms + 2 ms= 452 ms

- Exercise 4.13 a,b ( each part 20 points)

a. FLOP/s= 10 SIMD processors*1.5 GHz * 0.85 instr/cycle *0.7* 0.8 * 8 lanes per SIMD= 57.12 GFLOP/s

b. 1. Speedup= 10*16*0.8510*8*0.85= 2x Speedup

2. Speedup= 16*8*0.8510*8*0.85= 1.5x Speedup

3. Speedup= 10*8*0.9510*8*0.85= 1.117x Speedup

- Exercise 4.16 (20 Points)

Peak Single-Precision Floating-Point Throughput= 16 SIMDs*16 FPUs/SIMD*1.5 GHz= 384 GFLOP/s

Sustainable: ​100 GB/s4 B=100*109 B/s4 B=25*109=25 GFLOP/s

Since 384 GFLOP/s is greater than 25 GFLOP/s, the throughput is not sustainable given the memory band-width limitation.