Pledge Honor: I pledge my honor that I have abided by the Stevens Honor System.

CS 488 Midterm Exam: 100 Points; Time 120 minutes		Name:	Octavio Morales

1. (5 Points) State whether you will get memory alignment error for the following instructions and why? Assume x0 contains 0.

a. sw 	x3, 194(x0)

b. fld	f2,  264(x0)

ANSWER:

- You will get a memory alignment error because 194 is not divisible by 4 (194%4 is not equal to 0)

- You will not get an alignment error because 264 is divisible by 8 (264%8=0).

2. (5 Points) Describe in 2-3 sentences what has limited the rate of growth of the processors clock rate.

ANSWER: The growth of the clock rates is limited by heat and power consumption. This is because the small space would generate too much heat to be cooled well.

3. (5 Points). What is the main reason the simple five-stage RISC-V architecture used WB split cycle?

ANSWER: The main reason the simple five-stage RISC-V architecture used WB split cycle since the split cycle would help with handling data hazards better. In addition, WB reads and writes in the same cycle without conflict.

- (5 Points) What is the main reason the simple five-stage RISC-V architecture used separate memory for data (DM) and instruction (IM)?

ANSWER: Separating instruction memory (IM) and data memory (DM) avoids structural hazards since allowing the IF stage and the MEM stage to run simultaneously would cause structural hazards. This ensures smooth pipeline operation without stalls due to memory conflicts.

5. (5 Points) Intel is spending $7 billion to complete its Fab 42 facility for 7 nm chip technology, which provides good dimensional scaling from its current 14nm. Based on what you learned in this course, what four major changes do you expect to see between 14nm and the new 7 nm?

ANSWER: Between the 14nm and the new 7nm, I could expect a better overall performance with a higher clock rate, lower cost, a lesser power consumption, and greater production output since there will be less defects.

6. (15 Points) A model of Intel Red Dragon processor has a die size of 400 mm2 and an estimated defect rate of 0.04 per cm2. The process complexity factor is 13.5.

a. (8 Points) Assume the wafer yield is 100%, what is the die yield?

b. (7 Points). What is the cost of a chip if the cost of a wafers of diameter 300 mm is $7000?

ANSWER:

- Yield=1(1+Defect Rate* Die Area)Complexity Factor

Yield=1(1+0.04*4)13.5=17.416=0.1348

Yield= 0.1348

- π(0.5*Diameter)2Die Area-π*Diameter2*Die Area

π(0.5*300)2400-π*3002*400=143.42 (rounded down to 143 since you can't have part of a die per wafer)

$7000143*0.1348=$363.14

7. (15 Points). You are considering enhancing a processor by adding a GPU (Graphical Processing Unit) to run “View Me” program faster. When computation is run in GPU, it is 6 times faster than the normal mode of execution.

a. (10 Points). What percentage of “View Me” should run in GPU to achieve an overall speedup of 4?

b. (5 Points). What percentage of the “View Me” execution time is in GPU when the overall speedup of 		4 is achieved?

ANSWER:

- Using Amdahl’s Law: S=1(1-f)+f6=4

14=(1-f)+f6

5f6=0.75

f=0.90

90% of “View Me” should run in GPU to achieve an overall speedup of 4.

- f/s((1-f)+f/s)=0.9/6((1-0.9)+0.9/6)=0.6

60% of execution time is spent on GPU when the overall speedup of 4 is achieved.

8. (15 Points) Designers are considering two options for improving a system’s performance. They know 20% of computation is graphic processing.

a. (7 Points) Option 1: Increase the main memory. This will improve 60% of the graphic processing by a 			        factor of 4.

b. (8 Points) Option 2: Upgrade the GPU. This will improve the graphic processing by a factor of 2.

Which option is better? Justify your answer.

ANSWER:

Option 1: 11-.2*.6+0.25*0.2*0.6=1.083

Option 2:1(1-0.2+0.5*0.2)= 1.11

Option two is slightly better since the speedup is greater.

9. (15 Points). Consider an architecture that executes an instruction in seven steps:

IF   (INSTRUCTION FETCH)= 4

ID (INSTRUCTION DECODE)=1

AG (OPERAND ADDRESS GENERATOR)= 1

OL (OPERAND LOAD)= 4

EX (EXECUTION)= 1

OS (OPERAND STORE)= 4

UP (UPDATE PC)= 1

a. (5 Points) If each memory access (IF, OL, OS) takes 4 units of time to complete, while all other 			steps take 1 unit of time, what is the cycle time of this un-pipelined processor?

b. (5 Points) If we pipeline the processor in part “a” into a seven-stage pipeline, which adds an 				overhead of 2 units of time per latch, what would be the cycle time of this pipelined processor?

c. (5 Points) What is the speedup obtained by the pipeline?

ANSWER:

- (As shown above) 4+1+1+4+1+4+1= 16 cycles

- The cycle time is 6 units, as there are 4 pipelined units and 2 overhead units

- 166=2.667 is the speedup

10. (15 Points). Consider the following code segment:

flw	f0, 0(x0)	; load f0 from address 0+x0

Loop:   flw	f2, 0(x2)	; load f2 from address 0+x2

fmult	f2, f2, f0	; f2 = f2 * f0

addwi	x2, x2, 4	; x2 = x2 + 4

fsd	f2, -4(x2)	; store x2 at address -4+x2

subw	x4, x3, x2	; x4 = x3 – x2

bne	x4, x0, -24 	; branch to loop if x4!= 0 (i.e. set PC<- PC+offset)

Assume the initial value of x3 is x2 + 300.

a. (4 Points). What is the size of this code in IM and the size of its data in DM?

b. (8 Points) Show the timing diagram of this instruction sequence for the five stage RISC-V 			pipeline with full forwarding paths including register read and write in the same cycle, comparison		performed in ID stage and PC updated in ID as well. Assume branch is handled by predicting it as		taken. Table is provided.

c. (3 Points) How many cycles does this loop take to execute?

ANSWER:

a. 7 instructions* 4 bytes= 28 bytes (Code Size in IM)

304 bytes as stated in the initial value and 4 bytes in the first instruction (Size of Data in DM)

b.

| Instruction | CC 1 | CC 2 | CC 3 | CC 4 | CC 5 | CC 6 | CC 7 | CC 8 | CC 9 | CC 10 | CC 11 | CC 12 | CC 13 | CC 14 | CC 15 | CC 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| flw f0, 0(x0) | IF | ID | EX | DM | WB |  |  |  |  |  |  |  |  |  |  |  |
| Loop: flw f2, 0(x2) |  | IF | ID | EX | DM | WB |  |  |  |  |  |  |  |  |  |  |
| fmult f2, f2, f0 |  |  | IF | Stall | EX | DM | WB |  |  |  |  |  |  |  |  |  |
| addwi x2, x2, 4 |  |  |  |  | IF | ID | EX | DM | WB |  |  |  |  |  |  |  |
| fsd f2, -4(x2) |  |  |  |  |  | IF | ID | EX | DM | WB |  |  |  |  |  |  |
| subw x4, x3, x2 |  |  |  |  |  |  | IF | ID | EX | DM | WB |  |  |  |  |  |
| bne x4, x0, -24 |  |  |  |  |  |  |  | IF | Stall | EX | DM | WB |  |  |  |  |
| flw f2, 0(x2) |  |  |  |  |  |  |  |  |  | IF | ID | EX | DM | WB |  |  |

c. 9 cycles for the first iteration. Onward, the cycle is 8 each since there is overlap between instructions. Finally, the last iteration is 10 cycles due to the stalls.

300/4= 75 iterations (as shown above)

Total Cycles= 9+8*73+10=603 cycles