Homework 4: 100 Points

This Homework contains examples on the features of modern microarchitecture: Prediction, Speculation, multiple issue, dynamic scheduling, and Out-of-Order Execution.

Please submit your answers in an MS Word document and upload it in Canvas by the due date. I

Present your work neat, clear, organized, logical, and make your case; don’t leave it to our interpretation of your answer. There is 10% penalty per day for delay in submitting the homework.

- (10 Points) Suppose the following code is within a loop reading the values of x. Assume a Branch History Table of 1-bit predictor used. Use a table to show the content of the predictor buffers as values of x: 12, 18, 23, 40, 42, 44, 46, 48, 50 are read. Assume the two predictor buffers contain 0 and 1 initially.

If (x is odd) then		;Branch B1

print “O”

if (x is even) then		;Branch B2

print “E”

Calculate the prediction accuracy of this predictor.

| Value | Odd or Even | Predictor | Branch | Is It Right? | Updated Predictor |
| --- | --- | --- | --- | --- | --- |
| 12 | Even | 0 | B2 | Yes | 0 |
| 18 | Even | 0 | B2 | Yes | 0 |
| 23 | Odd | 0 | B1 | No | 1 |
| 40 | Even | 1 | B2 | No | 0 |
| 42 | Even | 0 | B2 | Yes | 0 |
| 44 | Even | 0 | B2 | Yes | 0 |
| 46 | Even | 0 | B2 | Yes | 0 |
| 48 | Even | 0 | B2 | Yes | 0 |
| 50 | Even | 0 | B2 | Yes | 0 |

Accuracy= 7/9= 0.77…

- (10 Points) Answer Problem 1 using a Branch History Table of 2-bit predictor. Assume the two predictor buffers contain 10 and 10 initially.

| Value | Odd or Even | Predictor | Prediction | Actual | Is It Right? | Updated Predictor |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | Even | 10 | Taken | Taken | Yes | 11 |
| 18 | Even | 11 | Taken | Taken | Yes | 11 |
| 23 | Odd | 10 | Taken | Not Taken | No | 01 |
| 40 | Even | 11 | Taken | Taken | Yes | 11 |
| 42 | Even | 11 | Taken | Taken | Yes | 11 |
| 44 | Even | 11 | Taken | Taken | Yes | 11 |
| 46 | Even | 11 | Taken | Taken | Yes | 11 |
| 48 | Even | 11 | Taken | Taken | Yes | 11 |
| 50 | Even | 11 | Taken | Taken | Yes | 11 |

Accuracy= 8/9= 0.88…

- (10 Points) Suppose the following loop iterates 99 times. Assume a 1-bit predictor is used. Calculate the prediction accuracy of this predictor. Assume the buffer contains 0 initially.

Loop:	LD F1, 0(R1)

……

……

BNEZ R2, Loop

| Iteration(s) | Predictor | Actual | Is It Right? | Updated |
| --- | --- | --- | --- | --- |
| 1 | 0 | Taken | No | 1 |
| 2 | 1 | Taken | Yes | 1 |
| 3-98 | 1 | Taken | Yes | 1 |
| 99 | 1 | Not Taken | No | 0 |

Accuracy= 97/99= 0.9797…

- (10 Points) Answer Problem 3 using a 2-bit predictor. Assume the buffer contains 00 initially.

| Iteration(s) | Predictor | Actual | Is It Right? | Updated |
| --- | --- | --- | --- | --- |
| 1 | 00 | Not Taken | No | 01 |
| 2 | 01 | Not Taken | No | 10 |
| 3 | 10 | Taken | Yes | 11 |
| 4-98 | 11 | Taken | Yes | 11 |
| 99 | 11 | Not Taken | No | 10 |

Accuracy= 97/99= 0.9797…

- (12 Points) Consider the code in Example 4, Chapter 3 slides. Assume a ten entry Branch History Table of 2-bit predictor used with initial values of 01. Show the prediction behavior if the following are read into the code via a loop:

aa=3 and bb=2, followed by aa=4 and bb=5, followed by aa=2 and bb=2

Calculate the overall prediction accuracy.

| Branch | Value | Predictor | Actual | Is It Right? | Updated |
| --- | --- | --- | --- | --- | --- |
| b1 | aa=3 | 01 | Not Taken | Yes | 01 |
| b2 | bb=2 | 01 | Taken | No | 10 |
| b3 | aa not equal to bb | 01 | Taken | No | 10 |
| b1 | aa=4 | 01 | Not Taken | Yes | 01 |
| b2 | bb=5 | 10 | Not Taken | No | 01 |
| b3 | aa not equal to bb | 10 | Taken | Yes | 11 |
| b1 | aa=2 | 01 | Taken | No | 10 |
| b2 | bb=2 | 01 | Taken | No | 10 |
| v3 | aa=bb | 11 | Not Taken | No | 10 |

Accuracy= 3/9= 0.33…

- (12 Points) a) Display a Branch Target Buffer for the code in Example 6. Assume 1- bit predictor for branches are initially 1, 0, 1.

| Branch Address | Predictor PC | Predictor Bit |
| --- | --- | --- |
| 2012 | 202 | 1 |
| 2036 | 2064 | 1 |

  - Now assume you execute the code again and this time all three branches are taken. Display the new BTB.

| Branch Address | Predictor PC | Predictor Bit |
| --- | --- | --- |
| 2012 | 2020 | 1 |
| 2024 | 2032 | 1 |
| 2036 | 2064 | 1 |

- (12 Points) State what takes place when each of the following instructions reach the top of  ROB and their operands are all available:

  - L.D	F2, 0(R6)

  - ADD.D	F2, F4, F6

  - S.D	F2, 0(R2)

  - DSUBU	R3, R3, #-8

  - BNEZ	R3, target  (Predicted Correctly)

  - BEQZ	R3, Loop     (Mispredicted)

| Instruction | Action at Top of ROB |
| --- | --- |
| L.D F2, 0(R6) | Value is written to F2- instruction commits. |
| ADD.D F2, F4, F6 | Result is written to F2- instruction commits. |
| S.D F2, 0(R2) | Value in F2 is stored to memory- store executes and instruction commits. |
| DSUBU R3, R3, #-8 | Result is written to R3- instruction commits. |
| BNEZ R3, target | Prediction was correct- branch commits and control flow continues normally |
| BEQZ R3, Loop | Prediction was wrong- flush previous instructions and update PC to correct target (ie the loop) |

- (12 Points) Exercise 3.1 of textbook.

| Line | Instruction | Latency |
| --- | --- | --- |
| 0 | fld f2, 0(Rx) | 4 |
| 1 | fmul.d f2, f0, f2 | 5 |
| 2 | fdiv.d f8, f2, f0 | 11 |
| 3 | fld f4, 0(Ry) | 4 |
| 4 | fadd.d f4, f0, f4 | 3 |
| 5 | fadd.d f10, f8, f2 | 3 |
| 6 | fsd f4, 0(Rx) | 2 |
| 7 | addi Rx, Rx, -8 | 1 |
| 8 | addi Ry, Ry, 8 | 1 |
| 9 | sub x20, x4, Rx | 1 |
| 10 | bnz x20, Loop | 2 |

4+5+11+4+3+3+2+1+1+1+2= 37 cycles

- (12 Points) Exercise 3.7 of textbook.

| Original Instruction | Renamed Instruction | Key Changes |
| --- | --- | --- |
| fld f2, 0(Rx) | fld T9, 0(Rx) | f2 -> T9 |
| fmul.d f5, f0, f2 | fmul.d T10, f0, T9 | f5 -> T10, f2 = T9 |
| fdiv.d f8, f0, f2 | fdiv.d T11, f0, T9 | f8 -> T11, f2 = T9 |
| fld f4, 0(Ry) | fld T12, 0(Ry) | f4 -> T12 |
| fadd.d f6, f0, f4 | fadd.d T13, f0, T12 | f6 -> T13, f4 = T12 |
| fadd.d f10, f8, f2 | fadd.d T14, T11, T9 | f10 -> T14, f8 = T11, f2 = T9 |
| sd f4, 0(Ry) | sd T12, 0(Ry) | f4 -> T12 |